# difflow: A JAX-Based Differentiable Framework for Chemical Process Simulation and Optimization

5 January 2026

# Summary

Chemical process simulation is fundamental to the design and optimization of industrial plants, yet most existing tools rely on finite-difference approximations or derivative-free methods for optimization. difflow is a Python framework that enables fully differentiable simulation of chemical processes using JAX [@jax2018github], providing exact gradients through automatic differentiation for gradient-based optimization, sensitivity analysis, and uncertainty quantification.

The framework implements common unit operations—reactors (CSTR, PFR, fed-batch), separators (flash, distillation, liquid-liquid extraction), and heat exchangers—with thermodynamic models ranging from ideal mixtures to cubic equations of state (Peng-Robinson, SRK). Flowsheets with recycle streams are solved using acceleration methods (Anderson, Wegstein), with gradients computed through implicit differentiation of the converged solution. A technoeconomic analysis module provides differentiable capital and operating cost correlations, enabling simultaneous technical and economic optimization.

# Statement of Need

Process systems engineers routinely optimize designs involving thousands of decision variables subject to complex constraints. Traditional simulators like Aspen Plus and HYSYS are powerful but closed-source, expensive, and provide only finite-difference gradients. Open-source alternatives such as DWSIM and COCO lack automatic differentiation entirely. IDAES [@idaes] built on Pyomo [@pyomo] offers equation-oriented modeling with symbolic derivatives but requires algebraic problem formulations rather than the procedural simulation approach familiar to most engineers.

difflow fills this gap by combining the intuitive sequential-modular simulation paradigm with JAX’s automatic differentiation. Users define unit operations as Python functions, compose them into flowsheets, and obtain exact gradients automatically. This enables:

- Gradient-based optimization of reactor volumes, separation stages, and operating conditions

- Sensitivity analysis computing ∂y/∂x for any output with respect to any input

- Uncertainty propagation through linear (Jacobian-based) or Monte Carlo methods

- Integration with machine learning frameworks for hybrid physics-ML models

The framework includes domain-specific plugins for biopharmaceutical manufacturing (bioreactors, chromatography, membrane filtration) and rare earth element separations (solvent extraction, precipitation), demonstrating extensibility to specialized applications.

# Key Features

Differentiable Unit Operations. All operations support jax.grad, jax.jacobian, and jax.jit:

# Create base params (V will be replaced)

base_params = CSTRParams(

V=1.0,  # Placeholder

rate_fn=rate_fn,

stoich=stoich,

rate_params=rate_params,

species_order=species_order,

)

def conversion(volume):

params = base_params.update(V=volume)

reactor = CSTR(params)

outlet, info = reactor(inlet_stream)

return info["conversion"]["A"]

# Exact gradient via automatic differentiation

dX_dV = jax.grad(conversion)(2.0)  # dConversion/dVolume

Flowsheets with Recycles. Implicit differentiation enables gradients through converged recycle loops without differentiating through iteration history.

Differentiable Economics. Capital costs, operating costs, NPV, and minimum selling price are all JAX-compatible, enabling optimization of economic objectives directly.

# Example: Sensitivity Analysis

The following demonstrates computing the sensitivity of reactor conversion to both volume and rate constant simultaneously:

def experiment(params):

"""Returns conversion as function of (V, A) parameters."""

cstr_params = base_params.update(**params)

reactor = CSTR(cstr_params)

outlet, info = reactor(inlet_stream)

return info["conversion"]["A"]

# Parameters to differentiate with respect to

params = {'V': 2.0, 'rate_params': {'A': 1000.0, 'Ea': 5000.0 * 8.314}}

# Compute gradient with respect to all parameters

sensitivities = jax.grad(experiment)(params)

print(f"dX/dV = {sensitivities['V']:.6f}")

print(f"dX/dA = {sensitivities['rate_params']['A']:.6f}")

These gradients match analytical derivatives exactly and are computed efficiently via reverse-mode automatic differentiation.

# Availability

difflow is available on GitHub at https://github.com/jkitchin/differentiable-flowsheets under the MIT license. Documentation and example notebooks covering reactors, separations, optimization, uncertainty quantification, and technoeconomic analysis are included.

# AI Disclosure

This software was developed with assistance from Claude Code (Anthropic), an AI coding assistant. The author directed the development, made architectural decisions, and reviewed all generated code.

# References